Automated Smart Parking System

System Features by Role and System Components

This document lists what the Admin and the Guard/Staff can do in the system, and what is inside the system itself (hardware, firmware, software, and database). It is a working draft for planning and can be adjusted as the design is finalized.

# 1. Roles

- Admin: manages the whole system, including accounts, slots, devices, settings, reports, and audit logs.

- Guard / Staff: monitors the parking area in real time, responds to alerts, and records incidents. Has limited, mostly view-level access.

# 2. Features by Role

Full = complete access. View only = can see but not change. Flag only = can raise a flag for the admin. — = no access.

| Category | Feature | Description | Admin | Guard / Staff |
| --- | --- | --- | --- | --- |
| Account and Access |  |  |  |  |
| Access | Login | Secure sign-in using username and password (passwords stored hashed). | Full | Full |
| Access | Change own password | Users can update their own credentials. | Full | Full |
| Access | Manage user accounts | Create, edit, disable, and reset accounts for guards and staff; assign roles. | Full | — |
| Live Monitoring |  |  |  |  |
| Monitoring | Live slot status board | Real-time map/grid of all slots showing green (vacant) and red (occupied). | Full | Full |
| Monitoring | Occupancy counters | Total, occupied, and available slots, updated automatically. | Full | Full |
| Monitoring | Sensor / device health | Shows each ESP32 and sensor as online or offline, with last-seen time. | Full | View only |
| Monitoring | Fault alerts | Warnings for offline sensors, stuck or invalid readings, and lost connection. | Full | View only |
| Slot and Device Management |  |  |  |  |
| Management | Add / edit / remove slots | Register parking slots and map each one to its sensor and microcontroller. | Full | — |
| Management | Sensor calibration settings | Set the detection distance threshold and sampling interval per slot or per sensor. | Full | — |
| Management | Set slot out of service | Mark a slot as under maintenance or blocked so it is excluded from availability counts. | Full | Flag only |
| Records and Reports |  |  |  |  |
| Records | Occupancy history | Searchable log of every slot status change with date and time. | Full | Today only |
| Records | Generate and export reports | Daily, weekly, and monthly usage, plus peak-hour analysis (PDF / CSV). | Full | Daily only |
| Records | Incident notes | Record parking violations or issues (e.g., illegal parking, overstaying); admin reviews all entries. | Full | Add / edit own |
| Records | System and audit logs | Track logins, setting changes, and account actions. | Full | — |
| Notifications |  |  |  |  |
| Alerts | Lot-full / slot-available alerts | Notifications when the lot becomes full or slots open up. | Full | Full |
| Alerts | Configure notification rules | Choose alert thresholds, channels, and recipients. | Full | — |
| System Settings |  |  |  |  |
| Settings | System configuration | Refresh interval, timeout limits, and general settings. | Full | — |
| Settings | Backup and restore | Export or restore the parking database. | Full | — |

# 3. What Is Inside the System

| Layer | Component | What it does | Suggested technology |
| --- | --- | --- | --- |
| Hardware (per slot) |  |  |  |
| Sensing | Ultrasonic sensor | Measures distance to detect whether a vehicle is in the slot. Waterproof type suits outdoor use. | JSN-SR04T (or HC-SR04 for indoor tests) |
| Control | Microcontroller | Reads the sensor, decides occupied or vacant, drives the LEDs, and sends status to the server over Wi-Fi. | ESP32 |
| Indicator | Red / green LEDs | Green = vacant, red = occupied, visible to drivers entering the lot. | 5 mm or high-brightness LEDs + resistors |
| Power | Power supply | Stable 5V supply per node, with optional backup for brief outages. | 5V adapter / power bank |
| Protection | Weatherproof enclosure | Shields electronics from rain, dust, and impact; reduces false readings. | IP-rated project box |
| Firmware (runs on the ESP32) |  |  |  |
| Logic | Reading and filtering | Takes several readings and uses the median to remove noise and false detections. | Arduino C++ |
| Logic | Occupancy decision | Compares distance to the threshold with a debounce delay so status does not flicker. | Arduino C++ |
| Logic | Status reporting | Sends status on change, plus a periodic heartbeat so the server knows the device is alive. | HTTP POST or MQTT |
| Logic | Reconnect and local fallback | Reconnects after Wi-Fi loss; LEDs keep working locally even when offline. | Arduino C++ |
| Network |  |  |  |
| Communication | Campus Wi-Fi / router | Carries data from the nodes to the server. Phone hotspot is the demo fallback. | Wi-Fi, HTTPS |
| Server and Software |  |  |  |
| Backend | REST API | Receives sensor data, validates it, updates the database, and serves the dashboard. | Node.js or PHP |
| Backend | Authentication and roles | Login, session handling, and role-based access for Admin and Guard. | Hashed passwords, session/JWT |
| Backend | Alert and notification service | Detects lot-full, slot-available, and offline events, then sends notifications. | Telegram bot or web push |
| Data | Database | Stores users, slots, devices, occupancy logs, incidents, and settings. | MySQL |
| Frontend | Web dashboard | Live slot board, reports, management pages, and settings, with a view per role. | HTML/CSS/JS (responsive) |
| Security |  |  |  |
| Protection | Access control and data safety | Restricts dashboard to authorized users, secures data in transit, and follows the Data Privacy Act (RA 10173). | HTTPS, input validation |

# 4. Database Tables

| Database table | Key fields | Purpose |
| --- | --- | --- |
| users | user_id, full_name, username, password_hash, role (admin/guard), status, created_at | Accounts and roles |
| slots | slot_id, slot_label, zone, status (vacant/occupied/out of service), last_updated | Current state of each slot |
| devices | device_id, slot_id, sensor_type, threshold_cm, last_seen, online_status | ESP32 and sensor registry, health tracking |
| occupancy_logs | log_id, slot_id, status, distance_cm, timestamp | Full history for reports and accuracy testing |
| incidents | incident_id, slot_id, description, reported_by, timestamp, status | Guard incident notes |
| notifications | notif_id, type, message, sent_to, timestamp | Record of alerts sent |
| audit_logs | audit_id, user_id, action, timestamp | Security and accountability trail |
| settings | key, value, updated_by, updated_at | System configuration |

# 5. Data Flow

- The ultrasonic sensor measures distance in each slot.

- The ESP32 filters the readings, decides occupied or vacant, and switches the red or green LED.

- On a status change (and on a regular heartbeat), the ESP32 sends the data to the API over Wi-Fi.

- The API validates the data, updates the slots table, and writes a record to occupancy_logs.

- The dashboard shows the new status to Admin and Guard, and the notification service sends alerts when needed.